THE CORPORATION OF THE TOWN OF CALEDON

BY-LAW NO. [By-law Number Inserted by Town]

Being a by-law to amend Comprehensive Zoning By-law 2006-50, as amended, with respect to Part of Lot 19, Concession 1, East of Hurontario Street (Chinguacousy), Part 5, 43R-32579; Parts 1 and 2, 43R-32579; Parts 3 and 4, 43R-40376, Town of Caledon, Regional Municipality of Peel, municipally known as 12211, 12213 and 12231 Hurontario Street.

WHEREAS Section 34 of the Planning Act, as amended, permits the councils of local municipalities to pass zoning by-laws for prohibiting the use of land or the erecting, locating or using of buildings or structures for or except for such purposes as may be set out in the by-law;

AND WHEREAS the Council of The Corporation of the Town of Caledon considers it desirable to pass a zoning by-law to permit the use of Part of Lot 19, Concession 1, East of Hurontario Street (Chinguacousy), Part 5, 43R-32579; Parts 1 and 2, 43R-32579; Parts 3 and 4, 43R-40376,Town of Caledon, Regional Municipality of Peel, for residential use purposes.

NOW THEREFORE the Council of The Corporation of the Town of Caledon enacts that By-law 2006-50 as amended, being the Comprehensive Zoning By-law for the Town of Caledon, shall be and is hereby amended as follows:

- The following is added to Table 13.1:

| Zone Prefix | Exception Number | Permitted Uses | Special Standards |
| --- | --- | --- | --- |
| R1 | AAA | - Apartment Accessory - Day Care, Private Home - Dwelling, Detached - Home Occupation | Finished Grade For the purposes of this zone, Finished Grade, with reference to a building, shall be calculated using the average elevation of the finished surface of the ground where it meets the exterior of the front of such building, including an attached garage but not including building encroachments. Lot Frontage In the case of a corner lot, the lot frontage shall be calculated as if the front and exterior side lot lines were extended to their point of intersection. Porch For the purposes of this zone, Porch shall mean a platform with or without foundation or cold cellar, extending from an exterior wall of a building and having at least 50% of one side of the vertical planes forming the perimeter unobstructed in any manner except by railings and stairs with access to grade. Access Regulations For the purpose of this zone, Sections 4.3.3 (minimum entrance setback) and 4.3.4 (minimum entrance separation) shall not apply. Accessory Building Location For the purpose of this zone, an accessory building or structure in a rear yard shall be located a minimum of 0.6m from any lot line. |

| Zone Prefix | Exception Number | Permitted Uses | Special Standards |
| --- | --- | --- | --- |
|  |  |  | Model Homes Notwithstanding 4.24.4 (Model Homes and Temporary Sales Structures), for the purpose of this zone, the maximum number of model homes permitted on lands which have received draft plan of subdivision approval shall not exceed 10% of the total number of dwelling units within the draft- approved plan of subdivision, up to a maximum of 10 model homes. Sight Triangles For the purposes of this zone Section 4.38 (Sight Triangles) shall not apply. Size of Parking Spaces For the purpose of this zone, the minimum size of a parking space shall be 2.75m in width and 5.5m in length. Zone Standards Lot Area (minimum) N/A Lot Frontage (minimum) interior lot: 9.1m corner lot: 10.3m Front Yard (minimum) to a main building: 2.5m to a vehicular door of a private garage: 5.5m Interior Side Yard (minimum) one side 0.6m other side 1.2m Exterior Side Yard (minimum) to a main building: 2.0m to a vehicular door of a private garage: 5.5m to a chamfer or rounding on a corner lot: 0.0m Rear Yard (minimum) 6.0 m Backyard Amenity Area (minimum): N/A Building Height (maximum) 13.0 m Building Area (maximum) N/A Landscaped Area (minimum) 30% Permitted Encroachments Notwithstanding the provisions of Section 4.29 (Permitted Encroachments), for the purpose of this zone, the following encroachment provisions shall apply: Bay, Box or Bow Windows with or without foundations shall be permitted to encroach a maximum 1.0m into a required front, exterior side or rear yard. Covered or Uncovered Porch or Balcony, Canopy or Portico shall be permitted to encroach a maximum: - 2.0m into a required front yard; - 2.5m into a required rear yard; - 1.5m into a required exterior side yard; - 0.6m into a required interior side yard, provided a minimum setback of 0.6m is maintained to an interior side lot line. Covered or Uncovered Steps or Stairs, Ramp or Barrier-free Access Feature not associated with a Deck shall be permitted to encroach a maximum: - 2.5m into a required front or rear yard - 2.0m into a required exterior side yard Deck ≥ 0.75m in height shall be permitted to encroach a maximum 3.0m inclusive of any stairs, ramp or barrier-free access feature into a required rear yard. Deck < 0.75m in height shall be permitted to encroach a maximum 0.6m into any required yard, provided that a minimum setback of 0.6m is maintained to the lot line. Eaves, Sills, Cornices, Parapets, or other similar Ornamental Architectural features shall be permitted to encroach a maximum 0.6m extending from: - a main building wall; b) a bay, box or bow window; or c) a covered or uncovered porch or balcony into a required yard, provided that: - a minimum setback of 0.6m is maintained to a lot line; and - in the case of eaves, a minimum setback of 0.2m is maintained to an interior side lot line Steps or Stairs, Ramp, Barrier-free access feature and Deck in a private garage shall be permitted to encroach a maximum of 0.5m into a required parking space. |
| R1 | BBB | - Apartment Accessory - Day Care, Private Home - Dwelling, Detached - Home Occupation | Finished Grade For the purposes of this zone, Finished Grade, with reference to a building, shall be calculated using the average elevation of the finished surface of the ground where it meets the exterior of the front of such building, including an attached garage but not including building encroachments. Lot Frontage In the case of a corner lot, the lot frontage shall be calculated as if the front and exterior side lot lines were extended to their point of intersection. Porch For the purposes of this zone, Porch shall mean a platform with or without foundation or cold cellar, extending from an exterior wall of a building and having at least 50% of one side of the vertical planes forming the perimeter unobstructed in any manner except by railings and stairs with access to grade. Access Regulations For the purpose of this zone, Sections 4.3.3 (minimum entrance setback) and 4.3.4 (minimum entrance separation) shall not apply. Accessory Building Location For the purpose of this zone, an accessory building or structure in a rear yard shall be located a minimum of 7.0m from the rear lot line and 0.6m from a side lot line. Model Homes Notwithstanding 4.24.4 (Model Homes and Temporary Sales Structures), for the purpose of this zone, the maximum number of model homes permitted on lands which have received draft plan of subdivision approval shall not exceed 10% of the total number of dwelling units within the draft- approved plan of subdivision, up to a maximum of 10 model homes. Sight Triangles For the purposes of this zone Section 4.38 (Sight Triangles) shall not apply. Size of Parking Spaces For the purpose of this zone, the minimum size of a parking space shall be 2.75m in width and 5.5m in length. Zone Standards Lot Area (minimum) N/A Lot Frontage (minimum) interior lot: 9.1m corner lot: 10.3m Front Yard (minimum) to a main building: 2.5m to a vehicular door of a private garage: 5.5m Interior Side Yard (minimum) one side 0.6m other side 1.2m Exterior Side Yard (minimum) to a main building: 2.0m to a vehicular door of a private garage: 5.5m Rear Yard (minimum) 10.0 m Backyard Amenity Area (minimum): N/A Building Height (maximum) 13.0 m Building Area (maximum) N/A Landscaped Area (minimum) 30% Permitted Encroachments Notwithstanding the provisions of Section 4.29 (Permitted Encroachments), for the purpose of this zone, the following encroachment provisions shall apply: One-Storey component of a Main Building: - 3.5m into a required rear yard up to a maximum of 60% of the width of the Lot Frontage. - Where a main building encroaches into the rear yard, the minimum interior side yard shall be 0.6m Bay, Box or Bow Windows with or without foundations shall be permitted to encroach a maximum 1.0m into a required front or exterior side yard. Covered or Uncovered Porch or Balcony, Canopy or Portico shall be permitted to encroach a maximum: - 2.0m into a required front yard; - 2.5m into a required rear yard; - 1.5m into a required exterior side yard; - 0.6m into a required interior side yard, provided a minimum setback of 0.6m is maintained to an interior side lot line. Covered or Uncovered Steps or Stairs, Ramp or Barrier-free Access Feature not associated with a Deck shall be permitted to encroach a maximum: - 2.5m into a required front or rear yard - 2.0m into a required exterior side yard Deck ≥ 0.75m in height shall be permitted to encroach a maximum 3.0m inclusive of any stairs, ramp or barrier-free access feature into a required rear yard. Deck < 0.75m in height shall be permitted to encroach a maximum 4.7m into a required rear yard, provided that a minimum setback of 0.6m is maintained to the side lot line. Eaves, Sills, Cornices, Parapets, or other similar Ornamental Architectural features shall be permitted to encroach a maximum 0.6m extending from: - a main building wall; b) a bay, box or bow window; or c) a covered or uncovered porch or balcony into a required yard, provided that: - a minimum setback of 0.6m is maintained to a lot line; and - in the case of eaves, a minimum setback of 0.2m is maintained to an interior side lot line Steps or Stairs, Ramp, Barrier-free access feature and Deck in a private garage shall be permitted to encroach a maximum of 0.5m into a required parking space. |
| R1 | CCC | - Apartment Accessory - Day Care, Private Home - Dwelling, Detached - Home Occupation | Finished Grade For the purposes of this zone, Finished Grade, with reference to a building, shall be calculated using the average elevation of the finished surface of the ground where it meets the exterior of the front of such building, including an attached garage but not including building encroachments. Lot Frontage In the case of a corner lot, the lot frontage shall be calculated as if the front and exterior side lot lines were extended to their point of intersection. Porch For the purposes of this zone, Porch shall mean a platform with or without foundation or cold cellar, extending from an exterior wall of a building and having at least 50% of one side of the vertical planes forming the perimeter unobstructed in any manner except by railings and stairs with access to grade. Access Regulations For the purpose of this zone, Sections 4.3.3 (minimum entrance setback) and 4.3.4 (minimum entrance separation) shall not apply. Accessory Building Location For the purpose of this zone, an accessory building or structure in a rear yard shall be located a minimum of 8.0m from the rear lot line and 0.6m from a side lot line. Model Homes Notwithstanding 4.24.4 (Model Homes and Temporary Sales Structures), for the purpose of this zone, the maximum number of model homes permitted on lands which have received draft plan of subdivision approval shall not exceed 10% of the total number of dwelling units within the draft- approved plan of subdivision, up to a maximum of 10 model homes. Sight Triangles For the purposes of this zone Section 4.38 (Sight Triangles) shall not apply. Size of Parking Spaces For the purpose of this zone, the minimum size of a parking space shall be 2.75m in width and 5.5m in length. Zone Standards Lot Area (minimum) N/A Lot Frontage (minimum) interior lot: 9.1m corner lot: 10.3m Front Yard (minimum) to a main building: 2.5m to a vehicular door of a private garage: 5.5m Interior Side Yard (minimum) one side 0.6m other side 1.2m Exterior Side Yard (minimum) to a main building: 2.0m to a vehicular door of a private garage: 5.5m Rear Yard (minimum) 11.0 m Backyard Amenity Area (minimum): N/A Building Height (maximum) 13.0 m Building Area (maximum) N/A Landscaped Area (minimum) 30% Permitted Encroachments Notwithstanding the provisions of Section 4.29 (Permitted Encroachments), for the purpose of this zone, the following encroachment provisions shall apply: One-Storey component of a Main Building: - 3.5m into a required rear yard up to a maximum of 60% of the width of the Lot Frontage. - Where a main building encroaches into the rear yard, the minimum interior side yard shall be 0.6m Bay, Box or Bow Windows with or without foundations shall be permitted to encroach a maximum 1.0m into a required front or exterior side yard. Covered or Uncovered Porch or Balcony, Canopy or Portico shall be permitted to encroach a maximum: - 2.0m into a required front yard; - 2.5m into a required rear yard; - 1.5m into a required exterior side yard; - 0.6m into a required interior side yard, provided a minimum setback of 0.6m is maintained to an interior side lot line. Covered or Uncovered Steps or Stairs, Ramp or Barrier-free Access Feature not associated with a Deck shall be permitted to encroach a maximum: - 2.5m into a required front or rear yard - 2.0m into a required exterior side yard Deck ≥ 0.75m in height shall be permitted to encroach a maximum 3.0m inclusive of any stairs, ramp or barrier-free access feature into a required rear yard. Deck < 0.75m in height shall be permitted to encroach a maximum 4.7m into a required rear yard, provided that a minimum setback of 0.6m is maintained to the side lot line. Eaves, Sills, Cornices, Parapets, or other similar Ornamental Architectural features shall be permitted to encroach a maximum 0.6m extending from: - a main building wall; b) a bay, box or bow window; or c) a covered or uncovered porch or balcony into a required yard, provided that: - a minimum setback of 0.6m is maintained to a lot line; and - in the case of eaves, a minimum setback of 0.2m is maintained to an interior side lot line Steps or Stairs, Ramp, Barrier-free access feature and Deck in a private garage shall be permitted to encroach a maximum of 0.5m into a required parking space. |

- Schedule “A”, Zone Map 7 of By-law 2006-50, as amended is further amended for PART OF LOT 19 CONCESSION 1 EAST OF HURONTARIO STREET CHINGUACOUSY, PART 5, 43R32579; PARTS 1 AND 2, 43R32579; PARTS 3 AND 4, 43R40376, Town of Caledon, Regional Municipality of Peel, from Agricultural (A1) and Highway Commercial (CH) to Residential One - Exception AAA (R1-AAA), Residential One - Exception BBB (R1-BBB), Residential One - Exception CCC (R1-CCC) and Open Space (OS) in accordance with Schedule “A” attached hereto.

Read three times and finally passed in open Council on the

[XX] day of [XXXXXX], [2024].

Annette Groves, Mayor

Laura Hall, C